200 MW USED DIESEL POWER PLANT for Sale

(4x50 MW UNITS) MAN-ABB Engine Generator Sets
50Hz.  11kV  HFO  103rpm

Commissioned 1999

Executive Summary

Plant Overview: The plant is a 200-MW LSHS (low sulphur heavy stock) fuel (processed from the residue of indigenous crude). The plant is based on two-stroke diesel engine technology from MAN B&W, Germany. There are 4 Units of 50 MW each and the plant is in running condition with minimum self-breakdown record. The plant was commissioned in the year 1999. The generator which is of ABB make generates electricity at 11KV. The engines and equipment are water cooled. A recirculation type cooling water system using fresh water with cooling towers is provided. The engines are housed in a building with adequate maintenance facilities like EOT Crane, hoists, etc. The main control room is also located in this building. The engine room is provided with mechanical ventilation system and the control room is air conditioned. The power plant has facilities for unloading, storage and supply of fuel oil to the engines. Separate tanks for HFO and LDO are provided. Water treatment facilities for cooling water system and to produce demineralized water for the waste heat recovery boilers are provided. Suitable Fire protection system to protect the equipment fire hazards is provided.

The main engines have been presently used only as a peaking power plant and are very well maintained. These engines have balance useful life of at least 20 years with proper maintenance as recommended by OEM.

Figure 1: Plant Map

- Technical Details

| Type of Station | Diesel |
| --- | --- |
| Station Capacity | 200MW (4x50 MW) |
| Fuel | LSHS & LDO |
| Transportation | Ship, Cross Country pipe line |
| Consumption | 810 Tonnes per day |
| Cooling Water Source | Sewage Treatment Plant |
| Water Consumption Requirement | 30 m3/hr |
| Chimney | RCC Chimney with Flue height 100 meters |
| Design Heat Rate | 7400BTU/kWhr |

- Main Engine

| Manufacturer | M/s Hyundai Heavy Industries Co. Ltd. |  |
| --- | --- | --- |
| Engine Type | HYUNDAI-MAN B&W,2-Stroke,single acting, cross-head, exhaust turbocharged type diesel engine |  |
| Model | 12K90MC-S |  |
| Number of Cylinder | 12 |  |
| Cylinder Bore | 900 | mm |
| Stroke | 2300 | mm |
| At Max. continuous rating |  |  |
| Output | 51480 | kW |
| Revolution | 103.4 | rpm |
| Mean effective Pressure | 17 | bar |
| Max.Pressure | 145 | bar |
| Mean piston speed | 145 | m/s |
| Net Weight | 1810 | ton |
| Direction of rotation | Clock wise, looking from aft |  |
| Cooling medium | Cylinder Jacket ------- | Fresh water |
|  | Piston ------- | Lubricating oil |
|  | Turbocharger ------- | Fresh water |
|  | Scav.air cooler ------- | Raw water |
| Starting System | Compressed air (Max. pressure 30 bar) |  |

- Plant Details

Major systems and equipment for mechanical, electrical, instrumentation & control and civil works are given below:

Fuel oil System

FO transfer Pump

FO Supply Pump

FO Circulating Pump

FO Purifiers

FO Filters

FO Heaters

FO Sludge Pump

FO Drain Pump

Oily Water Separater

Engine room Pit Pump

HFO Treatment room Pit Pump

LDO Supply Pump

Oily Water Separater

HSD Unloding pump

HSD Transfer pump

Lube Oil System

CLO Unloading pump

CLO transfer pump

MLO Unloading pump

MLO Pump

Main lube oil cooler

Used lub oil transfer pump

Lube Oil Sludge system

Lube Oil Filters

Lube Oil Purifiers

Cam Shaft Lub oil Pump

Piston rod filter

Water System

Make up water pump

Cooling water transfer pump

Cooling tower

Air coolers

Jacket cooling water cooler

Jacket water pump

Jacket water preheater pump

Jacket water preheater

Potable water pump

Soft water pump

Air System

Air Compressor

Air tanks

Air drier

Ventilation System

Viscous filters

Air suction Fans

Turbo charger

WHRB

Boilers

Condenser

Boiler Water feed pump

Hot well tank

Exhaust System

C&I

Engine Safety Panel

Digital Engine Control

Distributed Control system

UPS

Paging System

PA System

CCTV System

Master Clock System

Stack Gas Analysers

VMON System

Oil Mist Detectors

Viscosity Meters

Alpha Lubricators

PMI System

Weigh Bridge

I&C systems summary

Electrical

Alternators

Excitation System

Jacking Oil System

Generator Cooling system

Generator Transformers

Unit Auxiliary transformers

Station Transformers

Service Transformers

Lighting Transformers

STP Transformer

HT Motors

Battery Chargers

Switchyard

110kv SF6 Breakers

CVT's

CT's

Line Isolators

Lightening Arestors

Wave Trap

Post Insulator

Tariff Meters

RTU & SCADA

Sewage Treatment Plant

Biological Section

Chemical Section

Reverse Osmosis Section

Post Treatment section

Fire Fighting System

Water Hydrant System

High Velocity Water spray system

Medium Velocity Water Spray system

Foam System

Carbon dioxide-Total floding system

Carbon Dioxde-Spurt System

Protable Fire Extinguishers

Fire Detection & alarm System

Cranes

Water Hydrant System

The main engines have been presently used only as a peaking power plant and are very well maintained.

These engines have balance useful life of at least 20 years with proper maintenance as recommended by OEM.

- Plant Site Pictures

The pictures shown below are indicative in nature. Customers are advised to inspect the material for better understanding.

Figure 2: Fuel Oil System

Figure 3: Cooling Water System

Figure 4: Switchyard

Figure 5: Sewage Treatment Plant

Figure 5: Main Engine

Figure 6: Central Control Room

Figure 7: Fuel Oil Storage

Figure 8: Switchyard

Figure 9: Floor View

Figure 10: Transformer Plate